Spesifikasi Pemancar Short Wave (SW)

| No. | Spesifikasi | Parameter | Keterangan |
| --- | --- | --- | --- |
| A | General Characteristics |  |  |
| 1 | Transmitter Power Amplifier | Solid State |  |
| 2 | RF Output Power | ≥ 10 KW |  |
| 3 | RF Frequency Range | 3.2 – 18.0 Mhz |  |
| 4 | RF Output Impedance | 50 ohm |  |
| 5 | RF Load Vswr | Less than 2.0 |  |
| 6 | Frequency Stability | +- 2 x 10-7 (20 hz) |  |
| 7 | Mode of Operation | AM, AM witch CCM, DRM, Simulcast | DRM Ready |
| 8 | Overall Efficiency | ≥ 60 % |  |
| 9 |  |  |  |
| 10 | Cooling System | Air Cooling |  |
| 12 | Tuning Time | within 10 Seconds |  |
| 13 | Power Consumption | 200V to 500Vac, 3phase, 50/60Hz |  |
| 14 | RF Output Connector | 1-5/8” EIA Flange |  |
|  | Audio Characteristics |  |  |
| 1 | Input Connector | BNC or XLR |  |
| 2 | Audio Input Signals | -10dBm ~ +10dBm , 600ohm balance |  |
| 3 | Audio Frequency Response | ±1dB at 50Hz ~10KHz |  |
| 4 | Total Harmonic Distortion | <3.5 %, 50Hz – 7.5Khz, at m=30 – 80 % |  |
| 5 | Carrir Shift | ≥ 55 dB, reference level m = 100 % fm = 1 khz (unweighted) |  |
| B.2 | Enviromental Conditions |  |  |
| 1 | Ambient Tempetarure | -10℃ ~ + 50℃ |  |
| 2 | Humidity Range | less than 95% at 40℃ |  |

Spesifikasi Pemancar Amplitude Modulasion AM

| No. | Spesifikasi | Parameter | Keterangan |
| --- | --- | --- | --- |
| A | GENERAL CHARACTERISTICS |  |  |
| 1 | Transmitter Power Amplifier | Solid State Medium Wave |  |
| 2 | RF Output Power | ≥ 10 Kw |  |
| 3 | Mode of Operation | AM, AM with CCM, DRM, Simulcas |  |
| 4 | Efficiency | 86% typical at 10 kW |  |
| 5 | RF Frequency Range | 531 KHz – 1700 KHz | DRM Ready |
| 6 | RF Output Impedance | 50 Ω, unbalanced |  |
| 7 | RF Load Vswr | 1600 peak reflected watts (1.5:1 VSWR @ 10 kW, 100% modulation) results in instantaneous power shutback |  |
| 8 | Freq. Stability | ±2 ppm/year over temperature range |  |
| 9 | Cooling System | Forced Air |  |
| 10 | Power Supply Variation | ±10% voltage, 47 Hz to 63 Hz* |  |
| 11 | Power Consumption | 11.6 kW typical at 10 kW, 0% modulation; 17.4 kW typical at 10 kW, 100% modulation |  |
| 12 | RF Output Connector | 1-5/8" |  |
| B | AUDIO CHARACTERISTICS |  |  |
| 1 | Audio Input Impedance | balanced input without transformer >20 kΩ or 600 Ω internally jumper selectable |  |
| 2 | Digital Input | Dual AES-EBU Digital Audio inputs* adjustable from -30dBFS to 0dBFS for 100% modulation; I,Q over AES-EBU* with sample rate converter; I,Q over LVDS, 3 pairs, Clock, Data, frame sync |  |
| 3 | Frequency Response | +0.2 dB/-0.8 dB, 30 Hz to 10,000 Hz. |  |
| 4 | Total Harmonic Distortion (THD) | Better than 0.8% (THD), 30 Hz to 10,000 Hz at 95% modulation (typical) |  |
| 5 | Intermodulation Distortion | SMPTE 1:1 Ratio, 60Hz/7kHz, 95% Mod Peak - 0.5% @10 kW (typical); DIM-B, 2.96kHz/9kHz, 80% Mod Peak - 0.5% @ 10 kW |  |
| 6 | Carrier Shift | 0.5% or less |  |
| 7 | Hum and Noise | -65 dB or better below 100 % modulation (unweighted) -70 dB (Measured with IEC – A weighted lter) |  |

| C | Environmental & Electrical |  |
| --- | --- | --- |
| 1 | Temperature Range | 0°C to 45°C or Better |
| 2 | Hummidity Range | 0% to 95% non-condensing |
| 3 | Ac Input Supply | 208 Vac & 380 Vac, 3 phase |
| 4 | Transmission Line | Indoor , Rigid Line 1 5/8 |
| 5 | RF Switch Panel | 4 Port , 2 U link |
| 6 | Antenna Tuning Unit (ATU) | Power 55 KW; Frequency Spesific Factory Pre Tuning |
| 7 | Protection | Automatic Voltage Regulator 50 KVA; Isolation Transformer 50 KVA; Power On Timed Delay; Under/Over Voltage Protection; Surge Arester |